Adaptive LAG Load-Balance for Multicast

V1.0

Revision History

| Version | Date | Author | Comments |
| --- | --- | --- | --- |
| 1.0 | 3-Feb, 2026 | Pinaki Sarker | Created the separate ERD |

# Contents

Contents	3

1	Abstract	4

2	LAG Load balance for Multicast	4

3	Adaptive Load-Balance	7

# Abstract

This ERD (Engineering Requirement Document) is meant to define and describe the behaviour of NETGEAR’s unique feature on Managed Switch named ‘Adaptive LAG Load-balance for Multicast’.

# LAG Load balance for Multicast

There are numerous bugs filed where traffic is not balanced across the LAG members. Currently there is no granularity to select hashing parameters and schemes. This has been a bug since day 0. However, in order to bring more granularity and seamless zero touch experience, the following schemes along with adaptive load-balancing shall be provided.

Scheme-1: Non unicast global hashing

A command has to be provided so that user shall able to configure this mode irrespective of LAG type (manual or Auto).

(Config)#port-channel [auto] load-balance ?

0		     Non unicast global hashing

1                        Src MAC, VLAN, EType, incoming port

2                        Dest MAC, VLAN, EType, incoming port

3                        Src/Dest MAC, VLAN, EType, incoming port

4                        Src IP and Src TCP/UDP Port fields

5                        Dest IP and Dest TCP/UDP Port fields

6                        Src/Dest IP and TCP/UDP Port fields

7                        Enhanced hashing mode

Register: HASH_CONTROL

NON_UC_TRUNK_HASH_MOD_PORT_ENABLE=1

Scheme-2: Ability to set hashing based on source MAC/IP, destination MAC/IP, Source Port or Enhanced mode.

(Config)#port-channel [auto] load-balance 0 ?

1                        Source Port

2                        Source MAC/IP

3                        Destination MAC/IP

4                        Global Enhance mode

Register: HASH_CONTROL

NON_UC_TRUNK_HASH_MOD_PORT_ENABLE=1

NON_UC_TRUNK_HASH_SRC_ENABLE=1

NON_UC_TRUNK_HASH_DST_ENABLE=1

NON_UC_TRUNK_HASH_USE_RTAG7=1

Scheme-3: Ability to set advanced hashing parameters i.e. hash calculation.

(Config)#port-channel [auto] load-balance 0 4 <f3-f15>

Register: RTAG7_HASH_CONTROL_3

HASH_PRE_PROCESSING_ENABLE_A=1

HASH_A0_FUNCTION_SELECT=<0-15>

(Config)#port-channel [auto] load-balance 0 4 ?

f3		CRC16

f4		CRC16 and XOR1

f5		CRC16 and XOR2

f6		CRC16 and XOR4

f7		CRC16 and XOR8

f8		XOR16

f9		CRC16 Polynomial

f10		CRC16 LSB

f11		CRC16 MSB

Scheme-4: Ability to set advanced hashing parameters i.e. LBN.

(Config)#port-channel [auto] load-balance 0 4 lbn

(LBN)# port <port number> lbn <0-15>

(LBN)# hash offset <0-15>

Table/Register: PORT TABLE, RTAG7_HASH_TRUNK

dr " mod PORT <Port Id> 1 RTAG7_PORT_LBN=<0-15> "

dr " modreg RTAG7_HASH_TRUNK  SUB_SEL_NONUC=0x2 OFFSET_NONUC=<0-15> "

Switches with BCM56170 or any ASIC that supports RTAG7 Flow based hashing, following knobs must be exposed.

(Config)#port-channel [auto] load-balance 0 4 flow

(Flow)# even-parity <enable/disable>

(Flow)# concatenate hash (enable/disable)

(Flow)# subset  <0-7>

(Flow)# hash offset <6 bit>

dr " modreg RTAG7_HASH_SEL USE_FLOW_SEL_TRUNK_NONUC=1 "

Table: RTAG7_FLOW_BASED_HASH

EVEN_PARITY, CONCATENATE_HASH_FIELDS_TRUNK, SUB_SEL_TRUNK, OFFSET_TRUNK

dr " mod RTAG7_FLOW_BASED_HASH 0 1 EVEN_PARITY=1 SUB_SEL_TRUNK=1 OFFSET_TRUNK=0x7 CONCATENATE_HASH_FIELDS_TRUNK=1 "

(Config)#port-channel [auto] load-balance 0 4 port

(Port)# even-parity <enable/disable>

(Port)# concatenate hash (enable/disable)

(Port)# subset  <0-7>

(Port)# hash offset <6 bit>

dr " modreg RTAG7_HASH_SEL USE_FLOW_SEL_TRUNK_NONUC=0 "

Table: RTAG7_PORT_BASED_HASH

EVEN_PARITY, CONCATENATE_HASH_FIELDS_TRUNK_NONUC, SUB_SEL_TRUNK_NONUC, OFFSET_TRUNK_NONUC

dr " mod RTAG7_PORT_BASED_HASH 0 1 SUB_SEL_TRUNK_NONUC=0x1 EVEN_PARITY=0x1 CONCATENATE_HASH_FIELDS_TRUNK_NONUC=1 OFFSET_TRUNK_NONUC=0x2

A show command must be available to show the LAG id, LAG name, member ports, applied hashing scheme, and traffic status i.e. Balanced/Unbalanced. If there is no drop at any LAG member of a LAG or in simple word if there is no drop on a LAG then show command shall display ‘Balanced’ otherwise, ‘Unbalanced’ must be displayed against that very LAG. Example below:

(Config)#show port-channel [auto] load-balance state

Lag.    Channel                     Load     Mbr

Intf    Name            Link   Type Balance  Ports   		               State

------- --------------- ------ ---- -------- ------------------------------------ ----------

lag 1   ch1             Up     auto 7-lbn    1/0/43, 1/0/44, 1/0/45, 1/0/46 	Balanced

1/0/47

lag 2   ch2             Up     auto 7-f9     1/0/1, 1/0/2, 1/0/3, 1/0/4, 1/0/5	UnBalanced

lag 3   ch3             Up     auto 7-flow   1/0/6, 1/0/7, 1/0/8, 1/0/9, 1/0/10	UnBalanced

ep:enable

concat:enable

subset:3

hash offset:1

LBN Configuration:

Hash Offset: 4

Port 		Load Balance

Number		Number

------		------------

- 0

- 0

- 1

- 2

- 3

- 3

- 3

- 4

- 5

- 7

Well defined openAPIs must be provided so that all settings can be done via API call. Not to be mentioned an openAPI is required by which state of a LAG can be retrieved. All these configuration and display options must be available in Main UI and SNMP MIB.

# Adaptive Load-Balance

The following command will enable switch to apply adaptive load-balance to balance the LAG for multicast if it is found there is a drop on any member port of the LAG.

(Config)# [no] port-channel [auto] load-balance adaptive

When aforementioned command is enabled, switch shall continuously monitor the status of each LAG and if there is a drop identified, start issuing above schemes one by one with each variation and monitor the drop status. If drop is stopped, then apply or settle the settings and corresponding settings must be shown in running-config as the form or command and when saved, the configuration must be saved.

Even though the LAGs are well balanced, continuous monitoring shall be continued if command is enabled. At any given point of time, if LAGs are unbalanced then adaptive load-balancing shall be triggered till the LAGs are balanced.

Upon disabling the command, continuous monitoring of LAG load-balance thread shall be stopped.